CHAPTER 14   ALARM SYSTEMS ORDINANCE

ARTICLE I  TITLE, PURPOSE AND DEFINITIONS

Section 14-101  Purpose

The purpose of this Ordinance is to establish appropriate

guidelines for the installation of alarm systems for

notification of the Town of Freeport's Police and Fire

Departments.

Section 14-102  Definitions

For purposes of this Ordinance, certain terms or words used

herein shall be interpreted or defined as follows:

1.  Alarm System.  A system including any mechanism, equipment or device designed to automatically transmit a signal, message or warning from a private premises, including telephonic alarms systems designed to operate    automatically through the use of public telephone facilities to the Town of Freeport Public Safety Communications Center; or monitored by other private or public agencies which, in turn transmit a signal to the Town of Freeport Public Safety Communications Center.

ARTICLE II   PERMITS

Section 14-201  No person or business shall operate or  maintain an alarm system which by any method transmits a

signal, message or warning to the Police or Fire Departments  without first filing an application.

Section 14-202  Application for permits to install, maintain

or operate an alarm system shall be filed with the Chief of

each concerned department, or his designee, on forms supplied by the Town, together with an application fee of fifty-five dollars ($55.00) for residential service.  A fee of four-hundred and fifty dollars ($450.00) will be submitted to the Fire Department for non-residential installations.  A fee of fifty-five dollars ($55.00) will be submitted to the Police Department for non-residential burglar alarm installations.  Installations or monitoring will not be permitted until the application has been filed with the Freeport Dispatch Center and paid in full.  Said application shall set forth the name, address and telephone number of both the installer of the systems and the person or business on whose premises the system will be installed, as well as a description of the system and the location where it is proposed to be installed.  Installers are responsible for submission of said application and fee.

Section 14-203  The Chief of the concerned department shall

approve such application if he finds that:

1.  The use of said alarm system to transmit a signal,

message or warning to a designated Town Police

Department or Fire Department will not interfere with

the orderly conduct of Town business;

2.  The person installing the system maintains an adequate

service organization to repair, maintain or otherwise

service alarm systems sold or leased by him.  Any leased

monitoring company shall be UL listed and meet N.F.P.A.

Coded 71 standards or will be connected to municipal

fire alarm.

3.  The Police Chief or Fire Chief may impose other

reasonable conditions on the exercise of said permits.

The Town Manager shall retain final authority to approve

the reasonableness of any other conditions.

4.  The Chief of the concerned department shall have the

sole right to inspect or cause to be inspected by the

system installer any alarm system on premises where it

is intended to function prior to issuance of any permit

for operation of such a system, and he may inspect or

cause an inspection by the system installer or by a

qualified individual in alarm systems installation of

such system to be made at any time after the issuance of

a permit to determine whether it is being used in

conformity with the terms of the permit and the

provisions of this Ordinance.  Information related to

any alarm system shall be maintained in a confidential

manner and the release of such information by any member

of a concerned department shall be a violation of this

Ordinance.

5.  No person shall install, operate or maintain a

telephonic alarm system which automatically transmits a

signal, message or warning to the Town's Police or Fire

Department's telephone lines, except to such telephone

number or numbers as designated by the permit issued

under the provisions of this Ordinance.

6.  The Chief of the concerned department may revoke any

permit issued pursuant to the provisions of this

Ordinance, after giving written notice to the permit

holder and an opportunity for the permit holder to be

heard, if he determines that the alarm system installed

pursuant to said permit has been installed, maintained

or operated in violation of the provisions of this

Ordinance, or of any term or condition of said permit,

or for failure to pay any fee specified in Article III.

7.  A.  Any residential construction in units with one (1)

bedroom on one (1) floor will require a smoke

detector to be located in hall adjacent to bedroom

or sleeping quarters and one (1) in basement or

cellar.  Detectors will be wired together so one (1)

detector, when activated, will sound the other.  The

detectors will be hard wire 120V AC.

B.  In units with more than one (1) sleeping area, smoke

detectors will be installed to protect each bedroom

and one (1) in basement or cellar.  Detectors will

be wired together so one (1) detector, when

activated, will sound the other.  The detectors will

be hard wire 120V AC.

C.  In units two (2) stories above ground level,

detectors will be located in sleeping area or

adjacent hallway on ground floor and in cellar or

basement.  Detectors will be wired together so one

(1) detector, when activated, will sound the

other.  The detectors will be hard wire 120V AC.

ARTICLE III   TRANSMITTAL OF NON-EMERGENCY ALARMS

Section 14-301  Any permit holder whose system causes the

transmittal of a non-emergency police alarm more than two (2)times per calendar year shall pay a fee based upon the following scale:

3rd Alarm, $30.00

4th Alarm, $55.00

5th - 9th Alarm, $100.00

10th + Alarm, $150.00

In the case of the Fire Department, the fee shall be two hundred dollars ($200.00) for each instance of a non-emergency alarm in excess of three (3) such alarms per year.  Non-emergency alarms shall include, but shall not be limited to, malfunctioning equipment, or improperly monitored equipment such as to cause false alarms.

ARTICLE IV   REGISTRATION FEES

Section 14-401. An annual registration fee of thirty-five dollars ($35.00) for the administration and maintenance of the alarm registration record will be assessed at the beginning of each calendar year.

ARTICLE V  PENALTY

Section 14-501  Whoever violates any of the provisions of this Ordinance shall, upon conviction therefore, be punished by a fine of not more than two hundred dollars ($200.00).

ARTICLE VI   SAVINGS CLAUSE

Section 14-601  The invalidity of any provisions of this

Ordinance shall not affect the validity of any other provision.

Section 14-602  All fees associated with this Ordinance will

be deposited in a reserve account established by the Town and will be used for maintenance and expansion of the Town of Freeport Alarm System.

ARTICLE VII  EFFECTIVE DATE

Section 14-701

Updated with amendments April 1990, July, 2002, April 2005 and July 1, 2008.